# Protocol for testing Bluetooth loggers - Experiment 1

Aurore Ponchon

07/03/2022

## First experiment: measuring the signal strength at fixed distances

This first experiment is aimed at measuring the signal strength of mobile loggers at fixed distances to calibrate them and determine their position. It consists of testing at the same time 2 stationary loggers and 3 mobiles loggers.

# Material needed

- 2 poles

- 1 decameter

- tape

- 1 chronometer/watch

- 6 stationary loggers and their batteries (ID between 1 and 20)

- 2 charged mobile loggers (ID > 1000)

- ziplocks

# The stationary loggers

They are placed on a fixed pole, 2 at the top (1.8m), 2 in the middle (1.2m) and 2 towards the ground (40cm). Each logger at the same height is placed opposedly, so that one logger can directly detect mobile loggers while the other detect it from behind. Write down the ID and position of each logger. They have to be set to scan every 10 seconds (selecting overall efficiency), scan must be on and must be hidden from other loggers so that they are not detected. Check the time given to make sure it is updated and roughly the same among loggers (especially when gateway is off).

# The mobile loggers

They are placed on the ground in ziplocks individually and they all face as much as possible the stationary loggers. They have to be set to scan every 10 seconds. Any other option is off (no scanning nor hidden from other loggers). Check the time given to make sure it is updated and roughly the same among loggers (especially when gateway is off).

# The protocol

Start by placing the pole with mobile loggers at 10 cm from the fixed pole with stationary loggers. Wait 90-120s and move to 50cm, and then wait for 60sec, 1m, 1.5m etc until the loggers don’t get detected anymore. This can be checked on the ap. The stationary loggers (numbered between 1 and 100), should not record new logs when mobile loggers are out of reach. Each time we move the distance, the time must be recorded so that we know when exactly the distance is changed.

# Results from first trial of experiment 1 run indoors 18/02/2022

A first test was run indoors due to bad weather. 2 stationary loggers were set on the ground (1,4) as well as 3 mobile loggers (1002,1003,1005). This experiment was a failure, as mobile loggers were not detected further than 50cm.

# Results from second trial of experiment 1 run outdoors 21/02/2022

The stationary loggers (14,17,11,16,1,19) were placed on a pole at 40cm, 1.2m and 1.8m and 2 mobile loggers (1002 and 1020, only one with a detached battery) were moved along the meter. However, this trial was a failure due to the faulty gateway. Indeed, although data were regularly downloaded by the gateway while running the trial, the lack of exact time overwrote the data and all data were 0…

# Results from third trial of experiment 1 run outdoors 22/02/2022

The first trial in the morning was not successful. The data were detected up to 7m but then, before the data were downloaded from the app on the phone, we switched off the stationary loggers, which immediately erased the data. SO LESSON LEARNED: DO NOT SWITCH OFF THE LOGGERS BEFORE THE DATA ARE DOWNLOADED EITHER THROUGH THE APP ON A PHONE OR THROUGH THE GATEWAY.

A second trial was conducted in the afternoon and this time, the data were carefully downloaded from the ap on the phone internal memory (Document folder) and then extracted to a computer for analysis. Mobile loggers 1002 and 1003 were deployed with 1003 having the battery detached. 1002 was no longer detected when trials started so the data only concern one mobile logger. Two additional tables were generated along the log tables: one with the logger names and their position (position) and another one with the distance and corresponding time intervals (dist).

# Results from fourth trial of experiment 1 run outdoors 23/02/2022

With the same stationary loggers but placed differently on the pole, we ran the same experiment as previous day (22-02-2022). 5 mobile loggers were prepared: 1002 and 1003 with detached batteries and 1006,1007,1008 with attached batteries. Only 3 mobiles loggers were detected: 1003, 1006 and 1008 with strongly bias towards 1003 which was detected much more than the 2 others. Conditions were cold and windy with rain arriving just at the end of the experiment.

# Results from fifth trial of experiment 1 run outdoors 24/02/2022

With the same stationary loggers but placed differently on the pole and with new batteries, we ran the same experiment as previous day (22/23-02-2022). 5 mobile loggers were prepared: 1003 and 1010 with detached batteries and 1006,1008,1009 with batteries replaced to avoid having it behind the antenna. Conditions were cold but not windy and sunny.

# Results from sixth trial of experiment 1 run outdoors 25/02/2022

With the same stationary loggers placed at the same place on the pole, we ran the same experiment as previous day (22/23-02-2022). 5 mobile loggers were prepared: 1003 and 1010 with detached batteries and 1006,1008,1009 with batteries replaced to avoid having it behind the antenna. Conditions were cold but not windy and sunny.

# Results from seventh trial of experiment 1 run outdoors 28/02/2022

With the same stationary loggers placed at the same place on the pole, we ran the same experiment as previous day (22/23-02-2022). 5 mobile loggers were prepared: 1003 and 1010 with detached batteries and 1006,1008,1009 with batteries replaced to avoid having it behind the antenna. Conditions were not cold but cloudy and experiment had to be stopped due to rain at 4.5m. 1003 was on a stick

# Results from eighth trial of experiment 1 run outdoors 01/03/2022

With half the same stationary loggers and 3 new stationary loggers placed on the pole, we ran the same experiment as previous day (22/23-02-2022). 5 mobile loggers were prepared: 1003, 1005, 1008, 1009, 1010 with detached batteries. Conditions were not cold but cloudy and experiment had to be stopped due to rain at 4.5m. 1003 was on the ground while 1008 and 1010 were on sticks

# Figures for stationary loggers according to tests

Now that the data are imported and merged, we can generate figures showing the performance of the different loggers and tests. Each line corresponds to a test day, each color represents whether the stationary logger was facing the mobiles loggers or was opposite and each single line represent one stationary loggers. Columns are grouped by height at which the stationary loggers were positioned. If all the stationary loggers were performing the same, test 3, 4 and 5, which are run with the same loggers and positions, should have given the same results, which is not the case. Another result is that stationary loggers with the antenna facing the mobile loggers do not always outperform stationary loggers with the antenna opposite to the mobile loggers and we don’t see an increase of counts with the vertical position of stationary loggers.

For the next graph, the data are grouped the same way: each line represents one test, each column represents a height and colors represent stationary logger ID. On the y axis, we have the number of logs detected from mobile loggers and on the x-axis, the distance at which the mobile loggers were detected.

# Figures for mobile loggers

After analyzing the effect of stationary loggers, we can now check the performance of mobile loggers from the same dataset. It seems that detection of mobile loggers by stationary loggers is variable. For example, 1003 is doing much better than other mobile loggers for all stationary loggers while others are less detected. On the contrary, stationary logger 17 seems to perform poorly. So high variability between mobile and stationary loggers loggers but not within loggers so much. Empty boxes mean the loggers were not tested or not detected at all, even by the app.

On next figure, each line represents a test, each column represent a stationary logger and within each panel, we have the number of logs of mobile loggers according to the distance from the stationary loggers. Each color represents a mobile logger.